- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

Convert +45 to binary

45 in decimal=32 + 8 + 4 + 1

Therefore    +45 = 0010 1101

Step 2: Convert −23 to 2’s complement

First represent +23

23 = 16 + 4 + 2 + 1

+23 = 0001 0111

Take 1’s complement:

1110 1000

Add 1:

1110 1001

Therefore,

−23 = 1110 1001

A. Addition: +45 + (−23)

0010 1101     (+45)

+ 1110 1001     (−23)

----------------

1 0001 0110

Discard the carry beyond the 8th bit:

0001 0110

Convert to decimal:

0001 0110 = 16 + 4 + 2 = 22

Therefore,+45 + (−23) = +22

There is no overflow because the operands have different signs.

B. Subtraction: +45 − (−23)

Subtraction in 2’s complement is performed as:

45 − (−23) = 45 + 23

We have:

+45 = 0010 1101

+23 = 0001 0111

Therefore:

0010 1101

+ 0001 0111

------------

0100 0100

0100 0100 = 68

Therefore,

+45 − (−23) = +68

There is no overflow, because +68 is within the signed 8-bit range:

−128 to +127

Overflow Detection

For signed 2’s complement addition, overflow occurs when:

- Two positive numbers produce a negative result, or

- Two negative numbers produce a positive result.

In 45 + (−23), the operands have different signs, so overflow cannot occur.

In 45 − (−23) = 45 + 23, both operands are positive and the result is +68, which is within the range −128 to +127.

Final Result

| Operation | Binary Result | Decimal Result | Overflow |
| --- | --- | --- | --- |
| +45 + (−23) | 00010110 | +22 | No |
| +45 − (−23) | 01000100 | +68 | No |

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

Generate:Gᵢ = Aᵢ · Bᵢ

Propagate:Pᵢ = Aᵢ ⊕ Bᵢ

The carry equation is:

Cᵢ₊₁ = Gᵢ + PᵢCᵢ

Consider an example:

A = 1011

B = 0110

Assume initial carry:

C₀ = 0

Step 1: Generate and Propagate Signals

From LSB to MSB:

| Bit | Aᵢ | Bᵢ | Gᵢ = AᵢBᵢ | Pᵢ = Aᵢ⊕Bᵢ |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 |
| 3 | 1 | 0 | 0 | 1 |

Step 2: Calculate Carries in Advance

The carry equations are:

C₁ = G₀ + P₀C₀

C₁ = 0 + (1)(0)

C₁ = 0

Next:

C₂ = G₁ + P₁C₁

C₂ = 1 + (0)(0)

C₂ = 1

Next:

C₃ = G₂ + P₂C₂

C₃ = 0 + (1)(1)

C₃ = 1

Next:

C₄ = G₃ + P₃C₃

C₄ = 0 + (1)(1)

C₄ = 1

Therefore, all carries are determined using combination logic without waiting for one full adder after another.

Step 3: Calculate Sum

The sum equation is:

Sᵢ = Pᵢ ⊕ Cᵢ

Therefore:

S₀ = 1 ⊕ 0 = 1

S₁ = 0 ⊕ 0 = 0

S₂ = 1 ⊕ 1 = 0

S₃ = 1 ⊕ 1 = 0

Therefore:

Sum = 0001

Carry = 1

So:

1011 + 0110 = 1 0001

Decimal verification:

11 + 6 = 17

17 = 10001₂

CLA vs Ripple Carry Adder

In a ripple carry adder, each carry must wait for the previous carry:

C₀ → C₁ → C₂ → C₃ → C₄

Therefore, the delay increases as the number of bits increases.

In a CLA, the carries are calculated simultaneously using generate and propagate signals.

| Feature | Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- | --- |
| Carry calculation | Sequential | In advance |
| Delay | Higher | Lower |
| Hardware complexity | Low | Higher |
| Speed | Slower | Faster |
| Suitable for | Simple circuits | High-performance processors |

Conclusion

CLA improves processor speed because it reduces carry propagation delay. Although it requires additional logic hardware, its faster carry calculation makes it useful in high-performance arithmetic units.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

Multiplicand = −6

Multiplier = +5

Using 4-bit representation:

−6 = 1010

+5 = 0101

Let:

- M = 1010 (multiplicand)

- −M = 0110

- A = 0000 (accumulator)

- Q = 0101 (multiplier)

- Q₋₁ = 0

- Number of iterations = 4

Booth decision:

| Q₀ Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

Step-by-Step Process

Initial

A = 0000

Q = 0101

Q₋₁ = 0

M = 1010

Iteration 1

Q₀Q₋₁ = 10

Therefore:

A = A − M

0000 − 1010 = 0110

Perform arithmetic right shift:

A = 0011

Q = 0010

Q₋₁ = 1

Iteration 2

Q₀Q₋₁ = 01

Therefore:

A = A + M

0011 + 1010 = 1101

Perform arithmetic right shift:

A = 1110

Q = 1001

Q₋₁ = 0

Iteration 3

Q₀Q₋₁ = 10

Therefore:

A = A − M

1110 − 1010 = 0100

Perform arithmetic right shift:

A = 0010

Q = 0100

Q₋₁ = 1

Iteration 4

Q₀Q₋₁ = 01

Therefore:

A = A + M

0010 + 1010 = 1100

Perform arithmetic right shift:

A = 1110

Q = 0010

Q₋₁ = 0

Final Product

Combine A and Q:

A Q = 1110 0010

Therefore:

Product = 11100010₂

Since the result is negative, convert it from 2’s complement.

1’s complement:

00011101

Add 1:

00011110

00011110₂ = 30

Therefore:

11100010₂ = −30

Hence:

−6 × +5 = −30

Advantages of Booth’s Algorithm

- It directly handles signed numbers.

- It uses 2’s complement representation.

- Consecutive 1s in the multiplier can be handled efficiently.

- It reduces unnecessary addition operations.

- Arithmetic right shifting preserves the sign.

- It is suitable for hardware implementation in processors.

Conclusion

Booth’s algorithm efficiently performs signed multiplication. For this problem, the final result is −30, represented as 11100010₂ in 8-bit 2’s complement.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

Given:

- Dividend = 13 = 1101₂

- Divisor = 3 = 0011₂

- Number of bits = 4

A. Restoring Division

Initial:

A = 0000

Q = 1101

M = 0011

| Step | After left shift | A − M | Decision | Final A | Q |
| --- | --- | --- | --- | --- | --- |
| 1 | A=0001, Q=1010 | 0001−0011 = negative | Restore, Q₀=0 | 0001 | 1010 |
| 2 | A=0011, Q=0100 | 0011−0011 = 0 | Q₀=1 | 0000 | 0101 |
| 3 | A=0000, Q=1010 | 0000−0011 = negative | Restore, Q₀=0 | 0000 | 1010 |
| 4 | A=0001, Q=0100 | 0001−0011 = negative | Restore, Q₀=0 | 0001 | 0100 |

Therefore:

Quotient = 0100₂ = 4

Remainder = 0001₂ = 1

So:

13 ÷ 3 = 4 remainder 1

B. Non-Restoring Division

Initial:

A = 0000

Q = 1101

M = 0011

| Step | Operation | A after operation | Q |
| --- | --- | --- | --- |
| 1 | Shift, A = A − M | −2 | 1010 |
| 2 | Shift, A = A + M | 0 | 0101 |
| 3 | Shift, A = A − M | −3 | 1010 |
| 4 | Shift, A = A + M | −2 | 0100 |

At the end, A is negative, so perform the final correction:

A = A + M

−2 + 3 = 1

Therefore:

Quotient = 0100₂ = 4

Remainder = 0001₂ = 1

Comparison

| Feature | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Restoration | Required whenever subtraction is negative | Avoided during intermediate steps |
| Operations | More add/subtract operations | Fewer correction operations |
| Hardware | More complex | More efficient |
| Speed | Relatively slower | Relatively faster |
| Complexity | Simpler conceptually | Slightly more complex control |
| Modern use | Less efficient | Often preferred for high-speed implementations |

Conclusion

Both algorithms produce:

13 ÷ 3 = 4 remainder 1

Non-restoring division is generally preferred because it avoids immediately restoring the partial remainder after every unsuccessful subtraction, thereby reducing unnecessary operations and improving efficiency.
:::

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Step 1: Convert 13.25 to Binary

Integer part:

13 = 1101₂

Fractional part:

0.25 = 0.01₂

Therefore:

13.25 = 1101.01₂

Normalize:

1101.01 = 1.10101 × 2³

Since the number is negative:

−13.25 = −1.10101 × 2³

A. IEEE 754 Single Precision

Single precision contains:

- Sign = 1 bit

- Exponent = 8 bits

- Fraction/Mantissa = 23 bits

- Bias = 127

Sign Bit

The number is negative:

Sign = 1

Exponent

Actual exponent = 3

Biased exponent:

3 + 127 = 130

130 in binary:

10000010

Fraction

From:

1.10101 × 2³

The leading 1 is implicit, so the fraction field is:

10101000000000000000000

Therefore:

Sign     Exponent    Fraction

1       10000010   10101000000000000000000

Complete 32-bit representation:

11000001010101000000000000000000

B. IEEE 754 Double Precision

Double precision contains:

- Sign = 1 bit

- Exponent = 11 bits

- Fraction = 52 bits

- Bias = 1023

Sign

1

Exponent

Actual exponent = 3

Biased exponent:

3 + 1023 = 1026

1026 in 11-bit binary:

10000000010

Fraction

Fraction from normalized number:

10101

The remaining positions are filled with zeros:

1010100000000000000000000000000000000000000000000000

Therefore:

Sign       Exponent       Fraction

1          10000000010    1010100000000000000000000000000000000000000000000000

Thus, −13.25 is represented exactly in both single and double precision because its binary fraction terminates.

C. Floating-Point Addition

Floating-point addition generally follows these steps:

Step 1: Compare Exponents

The exponents of the two floating-point numbers are compared.

Step 2: Align the Mantissas

The number with the smaller exponent is shifted right until both numbers have the same exponent.

Example:

1.xxxxx × 2^m

1.yyyyy × 2^n

The smaller exponent is adjusted so both numbers use the same power of 2.

Step 3: Add or Subtract Significands

After alignment, the significands are added when their signs are the same.

For different signs, the smaller magnitude is subtracted from the larger magnitude.

Step 4: Normalize

The result must be normalized into the form:

1.xxxxx × 2ⁿ

If the result is too large or too small in the significand, the decimal/binary point is shifted and the exponent is adjusted.

Step 5: Rounding

If more bits are produced than the available fraction field, the result must be rounded according to the IEEE 754 rounding mode.

The commonly used default is round to nearest, ties to even.

Step 6: Check Overflow and Underflow

If the exponent becomes too large, floating-point overflow may occur.

If it becomes too small, underflow may occur.

Precision Loss

Precision loss can occur when:

- Exponents are very different.

- A small number is added to a very large number.

- The result contains more fraction bits than can be stored.

- Rounding is required.

Single vs Double Precision

| Feature | Single Precision | Double Precision |
| --- | --- | --- |
| Total bits | 32 | 64 |
| Sign | 1 | 1 |
| Exponent | 8 | 11 |
| Fraction | 23 | 52 |
| Bias | 127 | 1023 |
| Precision | Lower | Higher |

Conclusion

IEEE 754 provides a standardized method for representing real numbers. −13.25 can be represented exactly because its binary representation terminates. Double precision provides a larger exponent range and more fraction bits, so it generally provides greater precision and reduces rounding-related errors compared with single precision.